Clinical Guideline for: The Transfer of a Level 3 Critical Care Patient to MRI

| Author of Clinical Guideline | Dr Helen Gilfillan, Consultant Anaesthetist, Audit Lead |
| --- | --- |
| Division/ Department responsible for Clinical Guideline | Critical Care |
| Contact details | 2474 |
| Version number | 1.0 |
| Replaces version number | N/A |
| Date written | 4/03/23 |
| Consultation undertaken with: | Dr Mark Davidson, Consultant Intensivist and Anaesthetist, Clinical Lead for ICU Dr Cathryn Matthews, Consultant Anaesthetist |
| Approving body and date approved | ICU SGG 07/06/2023 |
| Review date | 07/11/2025 |
| Expiry date | 07/06/2026 |
| Date document becomes live | Date document is published on Hub |

SUMMARY

This guideline outlines the basic staffing/resource conditions which should be met during the transfer of a level 3 critical care patient to MRI. It also includes an outline of the various steps of the process.

CONTENTS

1.	INTRODUCTION	3

2.	BACKGROUND………………………………………………………………   ……3

3. 	MAIN BODY OF GUIDANCE   ………………………………………………….....3

4. 	MONITORING COMPLIANCE WITH THIS POLICY      ……………………….4

5. 	APPENDIX 1: Checklist and Suggested Sequence for Safe Critical Care Patient Transfers to MRI………………………………………………………….5

# 1.	INTRODUCTION

This guideline aims to provide a framework on which to base training, support good clinical practice and maintain patient and staff safety during the transfer of a level 3 patient from critical care for MRI. It should also facilitate the efficiency of the process, reducing the likelihood that MRI scan cannot be completed satisfactorily.

# 2.	BACKGROUND

# MRI is a hazardous environment for both staff and patients and the transfer of a critical care patient requires meticulous planning and preparation. This process will involve a team of staff, including members of critical care and anaesthetics, theatres and radiology. In addition, the equipment involved is specific to the environment with conditions for safe usage that may be unfamiliar to some clinicians.

Terminology regarding equipment and implants in MRI scanner:

- MRI safe – contains no material that would cause a hazard at any field strength e.g. MRI safe trolley

- MRI Conditional – safe to scan under specific conditions detailed by manufacturer and based on testing e.g. MRI conditional monitoring (Phillips MR 400 monitor in use in this Trust)

- MRI unsafe – poses unacceptable risk to patients and staff if used in MR environment e.g. ferromagnetic gas cylinders

# 3.	BASIC CONDITIONS FOR SAFE TRANSFER

3.1	Patient and timing:

- Whenever possible, patients should be in a physiologically stable condition prior to transfer to the MRI suite.

- Scans should be planned to occur between 10am and 4pm, Monday to Friday.

- When MRI is necessary an unstable patient, or is required outside of routine hours, the increase in risk should be reflected in the seniority/experience of the accompanying team.

3.2	Minimum personnel:

- Transfer will require a minimum team of 2 people although 3 is preferred.

- If only 2 personnel are involved, both must have experience delivering safe critical care within the local MRI suite.

- The lead clinician for the transfer process should be a Consultant with experience working in the local MRI suite and familiarity with the specific equipment involved.

- An advanced anaesthetic or intensive care trainee may lead the transfer process if they have appropriate confidence and competence; they should be familiar with the local MRI suite and equipment, and the planned process must be discussed with a consultant before commencing the transfer.

- The remainder of the team will usually include the patient’s bedside nurse and any combination of ACCPs, senior ICU nursing staff, ODPs, ICU and anaesthetic doctors.

- Additional staff will be required if the patient needs a log roll.

- If it is intended that the anaesthetic machine will be used during the MRI procedure, it is strongly recommended an ODP or anaesthetist is part of the transfer team.

Equipment:

- If intended to be used, the anaesthetic machine and monitoring equipment must be checked prior to departure of the patient from ICU.

# 4.	MONITORING COMPLIANCE WITH THIS GUIDELINE

This will be monitored by the Audit Leads for Critical Care (Anaesthetics and ICU)

5. APPENDIX 1:

Checklist and Suggested Sequence for Safe Critical Care Patient Transfers to MRI

If the emergency theatre anaesthetic team are required to perform the transfer, liaise directly with them in Theatre 9 (ext 2409 or bleep 489), making contact as soon as possible to allow coordination with other theatre activity.

Contact MRI scan room to establish the time you are expected in the MRI suite and confirm time porters will arrive on the unit.

If the anaesthetic machine needs to be checked and set up, contact ODP floor coordinator on bleep 415 with the details of the patient, the scan time and clinician leading the transfer.

PREPARATION OF THE PATIENT ON ICU

Ensure all staff in the team have completed an MRI safety screening questionnaire.

Patient safety:

- Complete MRI patient safety checklist and discuss any problems with radiology staff prior to transfer.

- Remove any metallic items e.g. hair clips, glasses, jewellery, hearing aids, safety pins, zips in clothing etc.

Airway and Breathing:

- Establish patient on transfer ventilator approximately 30 mins before planned departure.

- Remove any inline suction apparatus.

- Ensure HME has capnography port for MRI capnograph connection

Infusions:

- Stop any non-essential infusions. A maximum of 3 infusions should be in progress during the transfer and imaging process i.e. propofol  +/- opioid, vasoactive agent. Consider bolus doses of opioids and muscle relaxants to avoid unneccessary infusions.

- Prepare pumps 2 pumps for each infusion running: 1 will be in progress on ICU and for transfer to MRI. A second pump should loaded with a new syringe attached to 4 infusion lines for use during the scan. Ensure lines purged to the end and secure ends with a bung.

- Label each syringe and the beginning and end of the infusion line. Prepare 2 syringes of each drug (1 for each pump). Anticipate a minimum of 90 minutes outside the unit: prepare spare ‘extra’ syringes for infusion in case of delays during transfer process.

Additional considerations:

- Identify an IV access line available for use to bolus medication or contrast.

- Stop NG feed and disconnect.

- CVP monitoring should be disconnected.

- Arterial line transducers should be changed to MRI safe equipment (including pressure bag) if planned to be used in scan room.

- Ensure all catheter bags are drained and empty.

- Added extras:

- Spare bungs and sterets to maintain sterility when disconnecting/reconnecting infusions.

- Saline flushes and drawing up needles.

- Plastic clamps for any chest drains in situ.

The Team Leader:

- Should prepare the drugs they would like available during the transfer, including those for bolus administration and label them appropriately e.g. muscle relaxants & emergency drugs

- Review an ABG performed with the patient established on transfer ventilation settings prior to departure.

- Ensure team introductions are made (where necessary) including the MRI experience of individuals involved.

Before departure from ICU:

- Contact the MRI scan control room to confirm the anaesthetic machine/monitoring is set up and available for use and inform them that you are departing.

SUGGESTED SEQUENCE IN THE MRI SUITE

On arrival to the MRI preparation area:

***ESSENTIAL: All team members to check they have no metallic items on their person. Any badges/pens/lanyards/mobile phones/bleeps to be left in the control room***

- Confirm patient remains stable

- Ask radiographers to extract MRI scan table from scan room and bring out to anteroom (if not possible the MRI safe trolley will be necessary for all transfers into and out of scan room)

- Transfer patient on to the MRI scan table

- Attach MRI monitoring and confirm connectivity with control room screen. Normal ECG electrodes must be removed and replaced with MRI conditional version

- Remove transfer monitoring

- Move to MRI scan room doorway

- Move running pumps to a suitable location e.g. on a drip stand immediately outside the scan room door.

- Take prepared pumps with extended infusion lines to control room, plug in and pass distal ends through the hole in the wall (where anaesthetic circuit also passes through).

- Switch pumps on and set up to run at correct rates

- Take infusion line ends to the patient just outside scan room and swap with current infusions – confirm via drug labels on the end of the infusion lines

- Start new pumps at the same rate. For vasopressors consider ‘double pumping’ rather than stopping and starting infusions to maintain cardiovascular stability.

- Ensure pumps running with no ‘high pressure alarms’ (pressure limit alarms may need to be adjusted to address the additional resistance from the extended lines.  If these are high – check lines to ensure they remain patent)

- Stop old infusion pumps, cap off lines for use at end of scan. Set to charge if possible, away from scan room.

- Consider using volatile anaesthetic for sedation to enable reduction in infusions during scan

If using the anaesthetic machine:

- 1 member of team to set up the anaesthetic machine and ventilator at appropriate settings for the patient.

- The anaesthetic breathing circuit will reach the patient in the anteroom.  Disconnect transfer ventilator and establish ventilation via anaesthetic machine – check A+B, chest rise, ETCO2 etc first before moving patient into scan room.

**LAST CHECK: ensure no MRI unsafe items on patient and/or table**

Transfer into MRI scanner:

- Move patient into scan room; pumps MUST stay outside of the scan room

- Check ventilation/breathing on anaesthetic machine after every move: chest rise, CO2 trace, oxygen saturations

- Bring MRI conditional monitor into scan room – DO NOT PLACE MONITOR BEYOND THE RED DOTTED LINE ON THE FLOOR

- Prepare patient for scan:

  - ear protection

  - airway secure with pilot balloon taped away from area to be scanned to avoid image distortion and movement

  - arms secured

  - adequate length on infusion lines

  - cables safe

  - IV access available if required for contrast

  - Anaesthetic breathing circuit secured to table

- Perform run through into the scanner check all lines/cables/tubing long enough and safe.

- Team exit scan room

- In control room, check monitoring working and all physiological parameters appropriate before starting the scan.

DURING MRI SCAN

- Monitor patient and ensure observations are being recorded

- Plug in pumps to charge during scan if possible

- Review infusions/drugs required for return transfer

***REMEMBER: in any emergency, get the patient out of the scan room to allow safe treatment. It is too easy to bring unsafe equipment into the room***

ONCE MRI SCAN HAS BEEN COMPLETED

- Essentially a reverse of the entry process

- Ensure transfer ventilator and original pumps set up and ready immediately outside of the scan room

- Radiographers to use scan table to transfer patient out of scan room.  Once outside scan room, transfer onto bed.

- Disconnect extended infusion lines and reconnect original lines (attached to pumps just outside scan room door) restarting pumps at the correct rate

- Disconnect anaesthetic machine breathing circuit and connect to transfer ventilator

- Check ventilation is adequate & observations remain stable

- Switch from MRI to transfer monitoring

- Check observations remain stable, infusions adequately supplied etc prior to leaving MRI scan area

- Return to ICU

The team leader is responsible for ensuring MRI monitoring and anaesthetic machine are cleaned and returned to original positions after scan process complete.